#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <DHT.h>

#include <ESP32Servo.h>

#include <TinyGPSPlus.h>

#include <WiFi.h>

#include <PubSubClient.h>

// =====================================================

// FLOWSHIELD NODE 1

// FLOOD / DRAINAGE MONITORING NODE

// =====================================================

// =====================================================

// GPS

// =====================================================

#define GPS_RX 16

#define GPS_TX 17

HardwareSerial GPSserial(2);

TinyGPSPlus gps;

// =====================================================

// WIFI + MQTT

// =====================================================

const char* WIFI_SSID = "Velxio-GUEST";

const char* MQTT_BROKER = "broker.hivemq.com";

const int MQTT_PORT = 1883;

const char* MQTT_TELEMETRY_TOPIC =

"flowshield/node1/telemetry";

const char* MQTT_LOCATION_TOPIC =

"flowshield/node1/command/location";

const char* MQTT_STATUS_TOPIC =

"flowshield/node1/status";

WiFiClient flowShieldNet;

PubSubClient flowShieldMQTT(flowShieldNet);

double mapLatitude = 0.0;

double mapLongitude = 0.0;

bool mapLocationValid = false;

unsigned long lastMQTTReconnect = 0;

unsigned long lastTelemetry = 0;

// =====================================================

// OLED

// =====================================================

#define OLED_SDA 21

#define OLED_SCL 22

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(

SCREEN_WIDTH,

SCREEN_HEIGHT,

&Wire,

-1

);

// =====================================================

// DHT22

// =====================================================

#define DHT_PIN 4

#define DHT_TYPE DHT22

DHT dht(

DHT_PIN,

DHT_TYPE

);

// =====================================================

// ULTRASONIC SENSORS

// =====================================================

#define UP_TRIG 5

#define UP_ECHO 18

#define DOWN_TRIG 19

#define DOWN_ECHO 32

// =====================================================

// ANALOG INPUTS

// =====================================================

#define INFLOW_PIN 34

#define DRAINAGE_PIN 35

#define GAS_PIN 36

// =====================================================

// RGB LED

// =====================================================

#define RED_PIN 25

#define GREEN_PIN 26

#define BLUE_PIN 27

// =====================================================

// BUZZER

// =====================================================

#define BUZZER_PIN 23

bool buzzerActive = false;

// =====================================================

// SERVO

// =====================================================

#define SERVO_PIN 13

Servo drainageServo;

// Servo positions

const int SERVO_CLOSED = 30;

const int SERVO_OPEN = 150;

// =====================================================

// BUTTONS

// =====================================================

#define MAINTENANCE_BUTTON 14

#define RESET_BUTTON 33

// =====================================================

// SYSTEM STATE

// =====================================================

bool maintenanceMode = false;

bool highWater = false;

// =====================================================

// RGB FUNCTIONS

// =====================================================

void rgbOff()

{

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, LOW);

}

void rgbRed()

{

digitalWrite(RED_PIN, HIGH);

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, LOW);

}

void rgbGreen()

{

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, HIGH);

digitalWrite(BLUE_PIN, LOW);

}

void rgbBlue()

{

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, HIGH);

}

void rgbYellow()

{

digitalWrite(RED_PIN, HIGH);

digitalWrite(GREEN_PIN, HIGH);

digitalWrite(BLUE_PIN, LOW);

}

// =====================================================

// BUZZER FUNCTIONS

// =====================================================

void buzzerOn(int frequency)

{

if (!buzzerActive)

{

tone(

BUZZER_PIN,

frequency

);

buzzerActive = true;

}

}

void buzzerOff()

{

if (buzzerActive)

{

noTone(BUZZER_PIN);

buzzerActive = false;

}

}

// =====================================================

// SERVO FUNCTIONS

// =====================================================

void closeDrainageGate()

{

drainageServo.write(

SERVO_CLOSED

);

Serial.println(

"SERVO: DRAINAGE GATE CLOSED"

);

}

void openDrainageGate()

{

drainageServo.write(

SERVO_OPEN

);

Serial.println(

"SERVO: DRAINAGE GATE OPEN"

);

}

// =====================================================

// ULTRASONIC DISTANCE

// =====================================================

float readDistanceCM(

int trigPin,

int echoPin

)

{

digitalWrite(

trigPin,

LOW

);

delayMicroseconds(2);

digitalWrite(

trigPin,

HIGH

);

delayMicroseconds(10);

digitalWrite(

trigPin,

LOW

);

long duration =

pulseIn(

echoPin,

HIGH,

30000

);

if (duration == 0)

{

return -1;

}

float distance =

duration * 0.0343 / 2.0;

return distance;

}

// =====================================================

// OLED

// =====================================================

void showOLED(

float upstream,

float downstream,

int inflow,

int drainage,

int gas

)

{

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(

SSD1306_WHITE

);

// TITLE

display.setCursor(

0,

0

);

display.println(

"FLOWSHIELD NODE 1"

);

// UPSTREAM

display.setCursor(

0,

12

);

display.print(

"UP: "

);

if (upstream < 0)

{

display.println(

"ERROR"

);

}

else

{

display.print(

upstream,

1

);

display.println(

" cm"

);

}

// DOWNSTREAM

display.setCursor(

0,

22

);

display.print(

"DOWN: "

);

if (downstream < 0)

{

display.println(

"ERROR"

);

}

else

{

display.print(

downstream,

1

);

display.println(

" cm"

);

}

// INFLOW

display.setCursor(

0,

32

);

display.print(

"INFLOW: "

);

display.println(

inflow

);

// DRAINAGE

display.setCursor(

0,

42

);

display.print(

"DRAIN: "

);

display.println(

drainage

);

// STATUS

display.setCursor(

0,

52

);

if (maintenanceMode)

{

display.print(

"MAINTENANCE"

);

}

else if (highWater)

{

display.print(

"HIGH WATER!"

);

}

else

{

display.print(

"STATUS: NORMAL"

);

}

display.display();

}

// =====================================================

// WIFI

// =====================================================

void connectFlowShieldWiFi()

{

if (

WiFi.status() ==

WL_CONNECTED

)

{

return;

}

WiFi.mode(

WIFI_STA

);

WiFi.begin(

WIFI_SSID

);

Serial.print(

"Connecting to Velxio WiFi"

);

unsigned long start =

millis();

while (

WiFi.status() != WL_CONNECTED &&

millis() - start < 10000

)

{

delay(250);

Serial.print(".");

}

Serial.println();

if (

WiFi.status() ==

WL_CONNECTED

)

{

Serial.print(

"WiFi connected. IP: "

);

Serial.println(

WiFi.localIP()

);

}

else

{

Serial.println(

"WiFi connection pending."

);

}

}

// =====================================================

// MQTT CALLBACK

// =====================================================

void flowShieldMQTTCallback(

char* topic,

byte* payload,

unsigned int length

)

{

if (

strcmp(

topic,

MQTT_LOCATION_TOPIC

) != 0

)

{

return;

}

char message[96];

unsigned int n =

min(

length,

(unsigned int)(sizeof(message) - 1)

);

memcpy(

message,

payload,

n

);

message[n] =

'\0';

double lat = 0.0;

double lng = 0.0;

if (

sscanf(

message,

"%lf,%lf",

&lat,

&lng

) == 2

)

{

if (

lat >= -90.0 &&

lat <= 90.0 &&

lng >= -180.0 &&

lng <= 180.0

)

{

mapLatitude =

lat;

mapLongitude =

lng;

mapLocationValid =

true;

Serial.print(

"MAP LOCATION: "

);

Serial.print(

mapLatitude,

6

);

Serial.print(

", "

);

Serial.println(

mapLongitude,

6

);

}

}

}

// =====================================================

// MQTT CONNECTION

// =====================================================

void ensureFlowShieldMQTT()

{

if (

WiFi.status() != WL_CONNECTED

)

{

return;

}

if (

flowShieldMQTT.connected()

)

{

return;

}

if (

millis() - lastMQTTReconnect < 3000

)

{

return;

}

lastMQTTReconnect =

millis();

char clientId[64];

snprintf(

clientId,

sizeof(clientId),

"FlowShieldNode1-%08lx",

(unsigned long)millis()

);

Serial.print(

"Connecting MQTT... "

);

if (

flowShieldMQTT.connect(

clientId

)

)

{

Serial.println(

"CONNECTED"

);

flowShieldMQTT.subscribe(

MQTT_LOCATION_TOPIC

);

flowShieldMQTT.publish(

MQTT_STATUS_TOPIC,

"{\"node\":1,\"status\":\"online\"}"

);

}

else

{

Serial.print(

"FAILED rc="

);

Serial.println(

flowShieldMQTT.state()

);

}

}

// =====================================================

// MQTT TELEMETRY

// =====================================================

void publishFlowShieldTelemetry(

float upstream,

float downstream,

int inflow,

int drainage,

int gas,

float temperature,

float humidity

)

{

if (

!flowShieldMQTT.connected()

)

{

return;

}

const float MAX_RAINFALL_MM_H =

250.0;

float rainfall =

((float)inflow / 4095.0)

* MAX_RAINFALL_MM_H;

if (

isnan(temperature)

)

{

temperature = 0.0;

}

if (

isnan(humidity)

)

{

humidity = 0.0;

}

const float SENSOR_HEIGHT_CM =

30.0;

float waterLevel =

0.0;

if (

upstream > 0

)

{

waterLevel =

SENSOR_HEIGHT_CM -

upstream;

if (

waterLevel < 0

)

{

waterLevel = 0;

}

if (

waterLevel >

SENSOR_HEIGHT_CM

)

{

waterLevel =

SENSOR_HEIGHT_CM;

}

}

float waterPercent =

(waterLevel /

SENSOR_HEIGHT_CM)

* 100.0;

bool gpsValid =

gps.location.isValid();

bool altitudeValid =

gps.altitude.isValid();

double telemetryLat =

0.0;

double telemetryLng =

0.0;

if (

mapLocationValid

)

{

telemetryLat =

mapLatitude;

telemetryLng =

mapLongitude;

}

else if (

gpsValid

)

{

telemetryLat =

gps.location.lat();

telemetryLng =

gps.location.lng();

}

double gpsAltitude =

altitudeValid

? gps.altitude.meters()

: -1.0;

char payload[700];

snprintf(

payload,

sizeof(payload),

"{"

"\"node\":1,"

"\"latitude\":%.6f,"

"\"longitude\":%.6f,"

"\"map_location\":%s,"

"\"gps_valid\":%s,"

"\"upstream_cm\":%.2f,"

"\"downstream_cm\":%.2f,"

"\"inflow_adc\":%d,"

"\"drainage_adc\":%d,"

"\"gas_adc\":%d,"

"\"temperature_c\":%.2f,"

"\"humidity_pct\":%.2f,"

"\"rainfall_mm_h\":%.2f,"

"\"water_level_cm\":%.2f,"

"\"water_level_pct\":%.2f,"

"\"gps_altitude_m\":%.2f,"

"\"altitude_valid\":%s,"

"\"high_water\":%s,"

"\"timestamp_ms\":%lu"

"}",

telemetryLat,

telemetryLng,

mapLocationValid

? "true"

: "false",

gpsValid

? "true"

: "false",

upstream,

downstream,

inflow,

drainage,

gas,

temperature,

humidity,

rainfall,

waterLevel,

waterPercent,

gpsAltitude,

altitudeValid

? "true"

: "false",

highWater

? "true"

: "false",

millis()

);

flowShieldMQTT.publish(

MQTT_TELEMETRY_TOPIC,

payload

);

}

// =====================================================

// SETUP

// =====================================================

void setup()

{

Serial.begin(

115200

);

// ===================================================

// OLED

// ===================================================

Wire.begin(

OLED_SDA,

OLED_SCL

);

if (

!display.begin(

SSD1306_SWITCHCAPVCC,

0x3C

)

)

{

Serial.println(

"OLED FAILED"

);

}

else

{

Serial.println(

"OLED OK"

);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(

SSD1306_WHITE

);

display.setCursor(

0,

0

);

display.println(

"FLOWSHIELD"

);

display.println();

display.println(

"NODE 1"

);

display.println(

"INITIALIZING..."

);

display.display();

// ===================================================

// DHT

// ===================================================

dht.begin();

// ===================================================

// ULTRASONIC

// ===================================================

pinMode(

UP_TRIG,

OUTPUT

);

pinMode(

UP_ECHO,

INPUT

);

pinMode(

DOWN_TRIG,

OUTPUT

);

pinMode(

DOWN_ECHO,

INPUT

);

// ===================================================

// ANALOG

// ===================================================

pinMode(

INFLOW_PIN,

INPUT

);

pinMode(

DRAINAGE_PIN,

INPUT

);

pinMode(

GAS_PIN,

INPUT

);

// ===================================================

// RGB

// ===================================================

pinMode(

RED_PIN,

OUTPUT

);

pinMode(

GREEN_PIN,

OUTPUT

);

pinMode(

BLUE_PIN,

OUTPUT

);

rgbOff();

// ===================================================

// BUZZER

// ===================================================

pinMode(

BUZZER_PIN,

OUTPUT

);

digitalWrite(

BUZZER_PIN,

LOW

);

// ===================================================

// BUTTONS

// ===================================================

pinMode(

MAINTENANCE_BUTTON,

INPUT_PULLUP

);

pinMode(

RESET_BUTTON,

INPUT_PULLUP

);

// ===================================================

// SERVO

// ===================================================

Serial.println(

"Attaching servo..."

);

drainageServo.setPeriodHertz(

50

);

drainageServo.attach(

SERVO_PIN,

500,

2400

);

Serial.println(

"Servo attached to GPIO 13"

);

// ===================================================

// SERVO STARTUP TEST

// ===================================================

Serial.println(

"SERVO TEST: 30 degrees"

);

drainageServo.write(

SERVO_CLOSED

);

delay(1000);

Serial.println(

"SERVO TEST: 90 degrees"

);

drainageServo.write(

90

);

delay(1000);

Serial.println(

"SERVO TEST: 150 degrees"

);

drainageServo.write(

SERVO_OPEN

);

delay(1000);

Serial.println(

"SERVO TEST: returning to 30 degrees"

);

drainageServo.write(

SERVO_CLOSED

);

delay(1000);

// ===================================================

// GPS

// ===================================================

GPSserial.begin(

9600,

SERIAL_8N1,

GPS_RX,

GPS_TX

);

// ===================================================

// MQTT

// ===================================================

flowShieldMQTT.setServer(

MQTT_BROKER,

MQTT_PORT

);

flowShieldMQTT.setBufferSize(

1024

);

flowShieldMQTT.setCallback(

flowShieldMQTTCallback

);

connectFlowShieldWiFi();

ensureFlowShieldMQTT();

// ===================================================

// STARTUP BUZZER TEST

// ===================================================

Serial.println(

"Testing buzzer..."

);

buzzerOn(

1000

);

delay(300);

buzzerOff();

// ===================================================

// STARTUP RGB TEST

// ===================================================

Serial.println(

"Testing RED"

);

rgbRed();

delay(500);

Serial.println(

"Testing GREEN"

);

rgbGreen();

delay(500);

Serial.println(

"Testing BLUE"

);

rgbBlue();

delay(500);

rgbOff();

Serial.println();

Serial.println(

"=============================="

);

Serial.println(

" FLOWSHIELD NODE 1 READY"

);

Serial.println(

" Servo: GPIO 13"

);

Serial.println(

" Buzzer: GPIO 23"

);

Serial.println(

"=============================="

);

}

// =====================================================

// LOOP

// =====================================================

void loop()

{

// ===================================================

// WIFI + MQTT

// ===================================================

connectFlowShieldWiFi();

ensureFlowShieldMQTT();

flowShieldMQTT.loop();

// ===================================================

// ULTRASONIC

// ===================================================

float upstream =

readDistanceCM(

UP_TRIG,

UP_ECHO

);

float downstream =

readDistanceCM(

DOWN_TRIG,

DOWN_ECHO

);

// ===================================================

// ANALOG INPUTS

// ===================================================

int inflow =

analogRead(

INFLOW_PIN

);

int drainage =

analogRead(

DRAINAGE_PIN

);

int gas =

analogRead(

GAS_PIN

);

// ===================================================

// DHT22

// ===================================================

float temperature =

dht.readTemperature();

float humidity =

dht.readHumidity();

// ===================================================

// GPS

// ===================================================

while (

GPSserial.available()

)

{

gps.encode(

GPSserial.read()

);

}

// ===================================================

// DETERMINE WATER CONDITION

// ===================================================

highWater =

(

upstream > 0 &&

upstream < 10

);

// ===================================================

// MAINTENANCE BUTTON

// ===================================================

if (

digitalRead(

MAINTENANCE_BUTTON

) == LOW

)

{

maintenanceMode =

true;

Serial.println();

Serial.println(

">>> MAINTENANCE MODE <<<"

);

rgbYellow();

openDrainageGate();

buzzerOn(

1500

);

delay(300);

buzzerOff();

// Keep gate open while button is pressed

while (

digitalRead(

MAINTENANCE_BUTTON

) == LOW

)

{

delay(20);

}

Serial.println(

"Maintenance button released"

);

maintenanceMode =

false;

}

// ===================================================

// RESET BUTTON

// ===================================================

if (

digitalRead(

RESET_BUTTON

) == LOW

)

{

Serial.println();

Serial.println(

">>> SYSTEM RESET <<<"

);

maintenanceMode =

false;

highWater =

false;

closeDrainageGate();

rgbGreen();

buzzerOff();

delay(500);

}

// ===================================================

// AUTOMATIC DRAINAGE CONTROL

// ===================================================

if (

!maintenanceMode

)

{

if (

highWater

)

{

// ===============================================

// HIGH WATER

// ===============================================

Serial.println();

Serial.println(

"!!! HIGH WATER DETECTED !!!"

);

rgbRed();

openDrainageGate();

buzzerOn(

1800

);

}

else

{

// ===============================================

// NORMAL WATER

// ===============================================

Serial.println(

"STATUS: NORMAL"

);

rgbGreen();

closeDrainageGate();

buzzerOff();

}

}

// ===================================================

// OLED

// ===================================================

showOLED(

upstream,

downstream,

inflow,

drainage,

gas

);

// ===================================================

// SERIAL MONITOR

// ===================================================

Serial.println();

Serial.println(

"------------------------------"

);

Serial.print(

"Upstream: "

);

if (

upstream < 0

)

{

Serial.println(

"NO ECHO"

);

}

else

{

Serial.print(

upstream,

2

);

Serial.println(

" cm"

);

}

Serial.print(

"Downstream: "

);

if (

downstream < 0

)

{

Serial.println(

"NO ECHO"

);

}

else

{

Serial.print(

downstream,

2

);

Serial.println(

" cm"

);

}

Serial.print(

"Inflow ADC: "

);

Serial.println(

inflow

);

Serial.print(

"Drainage ADC: "

);

Serial.println(

drainage

);

Serial.print(

"Gas ADC: "

);

Serial.println(

gas

);

Serial.print(

"Temperature: "

);

if (

isnan(temperature)

)

{

Serial.println(

"ERROR"

);

}

else

{

Serial.print(

temperature,

2

);

Serial.println(

" C"

);

}

Serial.print(

"Humidity: "

);

if (

isnan(humidity)

)

{

Serial.println(

"ERROR"

);

}

else

{

Serial.print(

humidity,

2

);

Serial.println(

" %"

);

}

// ===================================================

// GPS

// ===================================================

if (

gps.location.isValid()

)

{

Serial.print(

"Latitude: "

);

Serial.println(

gps.location.lat(),

6

);

Serial.print(

"Longitude: "

);

Serial.println(

gps.location.lng(),

6

);

}

else

{

Serial.println(

"GPS: Waiting for fix..."

);

}

// ===================================================

// SERVO DEBUG

// ===================================================

Serial.print(

"Servo position: "

);

if (

maintenanceMode

)

{

Serial.println(

"150 degrees (MAINTENANCE)"

);

}

else if (

highWater

)

{

Serial.println(

"150 degrees (HIGH WATER)"

);

}

else

{

Serial.println(

"30 degrees (NORMAL)"

);

}

// ===================================================

// MQTT TELEMETRY

// ===================================================

if (

millis() - lastTelemetry >= 1000

)

{

lastTelemetry =

millis();

publishFlowShieldTelemetry(

upstream,

downstream,

inflow,

drainage,

gas,

temperature,

humidity

);

}

// ===================================================

// RAINFALL DEBUG

// ===================================================

float rainfallNow =

((float)inflow / 4095.0f)

* 250.0f;

Serial.print(

"LIVE ADC -> INFLOW(GPIO34)="

);

Serial.print(

inflow

);

Serial.print(

"  DRAINAGE(GPIO35)="

);

Serial.print(

drainage

);

Serial.print(

"  RAINFALL="

);

Serial.print(

rainfallNow,

1

);

Serial.println(

" mm/h"

);

delay(1000);

}