Mathematical Approaches & Formulas

1. Rainfall Intensity Calculation

Rainfall intensity (I) measures the depth of rain accumulated over time, typically expressed in millimeters per hour (mm/hr). Using a tipping bucket rain gauge connected to an IoT microcontroller, intensity is calculated based on pulse interrupts:

I=N×CΔt×3600

- N: Number of bucket tips recorded in time window Δt.

- C: Calibration factor per tip (e.g., 0.2794 mm per tip).

- Δt: Measurement interval in seconds.

- 3600: Multiplier converting seconds to hours.

2. Water Level & Depth Measurement

Ultrasonic sensors (such as HC-SR04) measure distance to the water surface using time-of-flight. Water depth (H) inside a channel or riverbed is derived as:

D=vsound×t2

H=Hsensor-D

- vsound: Speed of sound in air (≈343 m/s at 20∘C).

- t: Round-trip echo duration in seconds.

- Hsensor: Fixed elevation height of the sensor above the empty river bed.

3. Spatial Distance (Haversine Formula)

To calculate direct geographical distance (d) between two GPS coordinates ϕ1,λ1 and ϕ2,λ2 on Earth:

a=sin2Δϕ2+cosϕ1⋅cosϕ2⋅sin2Δλ2

c=2⋅atan2a,1-a

d=R⋅c

- ϕ1,ϕ2: Latitudes in radians.

- Δϕ=ϕ2-ϕ1 and Δλ=λ2-λ1.

- R: Mean Earth radius (6371 km).

4. Uniform Cost Search (UCS) for Safe Region Routing

Uniform Cost Search is a priority-queue pathfinding algorithm that finds the lowest cumulative path cost from a flooded origin node to the nearest safe evacuation zone.

- Path Cost Function: The edge cost Cu,v between node u and adjacent node v incorporates distance du,v and dynamic hazard risk weighting:

Cu,v=du,v×1+α⋅Riskv

- Riskv: Normalized risk score between 0.0 (dry) and 1.0 (severely flooded).

- α: Hazard penalty multiplier preventing routes through flooded areas.

- Evaluation Order: The algorithm maintains a priority queue Q sorted by path cost gn, expanding node n with mingn until reaching a safe node where RiskSafe Zone=0.

5. Composite Flood Risk Index

The overall threat level at a sensing station is computed via a weighted linear combination:

Risk Index=w1⋅HHthreshold+w2⋅IIthreshold+w3⋅QQmax

- H,I,Q: Measured water depth, rainfall intensity, and flow discharge rate.

- w1,w2,w3: Normalized weighting factors such that w1+w2+w3=1.0.